To start building your company's core engine, we need to translate your sketches into a formal Feature Roadmap and a Mathematical Blueprint. This architecture uses your "Geometric First" philosophy to bypass the need for expensive LiDAR or high-end GPU-heavy AI.

Here is the outline of the features and the corresponding formulae to actualize your vision.

### 1. Feature: Auto-Leveling & "Virtual Horizon"

Goal: Ensure the "Virtual Ground" is always level, even if the user tilts the phone. This corrects the input for all subsequent distance math.

- The Blueprint: Use the phone's Gravity Vector (from the accelerometer) to define the vertical $Z$-axis and the Horizon Line.

- The Formula:

$$\theta_{pitch} = \arctan\left(\frac{A_y}{\sqrt{A_x^2 + A_z^2}}\right)$$

(Where $A_x, A_y, A_z$ are raw accelerometer values. This angle $\theta$ is used to "rotate" the camera's raw view into a stabilized reference frame before calculation.)

### 2. Feature: Dynamic Scale Recovery (The "Step" Ruler)

Goal: Automatically calculate your camera height ($H$) by watching your walking gait.

- The Blueprint: Use the "Vertical Oscillation" (bobbing) of the phone during a step to create a known "Vertical Baseline".

- The Formula:

$$H = \frac{\Delta y_{imu}}{\Delta \phi_{visual}}$$

(By comparing the physical displacement measured by the IMU ($\Delta y$) to the angular pixel shift of floor points ($\Delta \phi$), the algorithm "solves" for the unknown height $H$.)

### 3. Feature: The "Virtual Vertical Wall" (Distance to Suspended Objects)

Goal: Measure distance to a sign or a box on a table by projecting its "visible base" onto the floor.

- The Blueprint: From the object's bottom-most pixel, the algorithm "drops" a virtual line parallel to the Vertical Vanishing Point until it hits the ground-plane map.

- The Formula:

$$d = \frac{H}{\tan(\alpha)}$$

(Where $H$ is the height from Step 2, and $\alpha$ is the angle of depression from the horizon to the point where your "Virtual Wall" touches the ground.)

### 4. Feature: The "Rising Floor" (Object Height $X$)

Goal: Calculate exactly how high a sign is hanging or how tall a table is.

- The Blueprint: As sketched in your notes, the horizon line splits the triangle into two parts ($H \mp X$).

- The Formula:

$$X = H + (d \cdot \tan(\beta))$$

(Where $d$ is the distance from Step 3 and $\beta$ is the angle of elevation from the horizon up to the sign's base. If the object is below the horizon, $\beta$ is negative, solving for "Table Height".)

### 5. Feature: "Orange into Deep Blue" (Bird's Eye Mapping)

Goal: Transform the 2D camera view into a top-down 2D floor map for the user to navigate.

- The Blueprint: Apply a Homography Matrix to "flatten" the perspective.

- The Formula:

$$\begin{bmatrix} x' \\ y' \\ 1 \end{bmatrix} = \mathbf{H} \begin{bmatrix} u \\ v \\ 1 \end{bmatrix}$$

(Where $\mathbf{H}$ is a $3 \times 3$ matrix derived from your camera's focal length and tilt. It maps pixel coordinates $(u, v)$ to real-world floor coordinates $(x', y')$.)

### 6. Feature: Visual-Inertial Anchors (The "Snapchat" Stabilizer)

Goal: Use visual "pins" on the floor to stop the IMU from "drifting".

- The Blueprint: Use Sparse Optical Flow (tracking specific high-contrast dots on the ground) to verify the phone's movement.

- The Formula:

$$E = \sum \| \text{Pixel}_{track} - \text{IMU}_{prediction} \|^2$$

(The system minimizes this "Error" ($E$). If the IMU says you moved but the "Anchors" on the floor didn't budge, the system ignores the IMU drift and keeps your position locked.)

### Implementation Summary

| Feature | User Sketch Reference | Math Key |
| --- | --- | --- |
| Virtual Wall | "make a virtual wall... to get distance" | Projective Geometry |
| Rising Floor | "$H \mp X$" | Trigonometric Height Offset |
| Orange to Blue | "Rotate orange into deep blue" | Homography Matrix ($3 \times 3$) |
| Dynamic Scale | "Get perception of phone movement" | Gait/Oscillation Ratios |

This is your master "Code Plan." When you begin building, you will start with Feature 1 (Auto-Leveling) and Feature 2 (Dynamic Scale) to "Zero" the system before moving to the 3D measurements.

Shall we look at the basic Python logic for "Feature 3: The Virtual Wall" as our first coding block?